IPv4 Address Classes and Network/Host Identification Guide

Overview

This guide covers IPv4 address classification and how to identify network and host portions of IP addresses - fundamental skills for subnetting and network administration.

IPv4 Address Structure

Basic Format

- Four octets separated by dots: 192.168.1.100

- Each octet ranges from 0 to 255

- Each octet = 8 bits

- Total = 32 bits (4 × 8)

Two Parts of Every IP Address

- Network portion - Identifies the network segment

- Host portion - Identifies the specific device on that network

IPv4 Address Classes

Class Ranges (Based on First Octet Only)

| Class | First Octet Range | Default Subnet Mask | Network/Host Split | Usage |
| --- | --- | --- | --- | --- |
| A | 1-127 | 255.0.0.0 | N.H.H.H | Large networks |
| B | 128-191 | 255.255.0.0 | N.N.H.H | Medium networks |
| C | 192-223 | 255.255.255.0 | N.N.N.H | Small networks |
| D | 224-239 | No default mask | - | Multicast |
| E | 240-255 | No default mask | - | Experimental |

Important Notes:

- Only look at the first octet to determine class

- No zero allowed in first octet (starts at 1)

- Classes A, B, C are used for standard networking

- Classes D and E have no default subnet masks

Step-by-Step Identification Process

Step 1: Address Class Identification

Method:

- Look at first octet only

- Compare to class ranges

- Identify the class letter

Examples:

- 10.250.1.1 → First octet: 10 → Range 1-127 → Class A

- 150.10.15.0 → First octet: 150 → Range 128-191 → Class B

- 192.14.2.0 → First octet: 192 → Range 192-223 → Class C

Step 2: Default Subnet Mask Identification

Default Masks by Class:

- Class A: 255.0.0.0

- Class B: 255.255.0.0

- Class C: 255.255.255.0

Method:

- Identify the class (Step 1)

- Look up corresponding default subnet mask

- Write complete mask with dots

Example:

- 177.100.18.4 → Class B → Default mask: 255.255.0.0

Step 3: Network vs Host Identification

The Golden Rule:

- 255 in subnet mask = Network portion

- 0 in subnet mask = Host portion

Network Identification Examples:

Example 1: Class A Network

Address: 119.18.45.0
Class: A (first octet: 119)
Default mask: 255.0.0.0
Network portion: Where mask = 255 → Octet 1 only

Example 2: Class B Network

Address: 177.100.18.4
Class: B (first octet: 177)
Default mask: 255.255.0.0
Network portion: Where mask = 255 → Octets 1 and 2

Example 3: Class C Network

Address: 209.240.80.78
Class: C (first octet: 209)
Default mask: 255.255.255.0
Network portion: Where mask = 255 → Octets 1, 2, and 3

Host Identification Examples:

Example 1: Class A Host

Address: 10.15.123.50
Class: A (first octet: 10)
Default mask: 255.0.0.0
Host portion: Where mask = 0 → Octets 2, 3, and 4

Example 2: Class B Host

Address: 171.2.199.31
Class: B (first octet: 171)
Default mask: 255.255.0.0
Host portion: Where mask = 0 → Octets 3 and 4

Example 3: Class C Host

Address: 198.125.87.177
Class: C (first octet: 198)
Default mask: 255.255.255.0
Host portion: Where mask = 0 → Octet 4 only

Working with Custom Subnet Masks

When Given a Custom Mask:

Use the provided mask instead of the default mask

Example:

Address: 10.10.48.80
Given mask: 255.255.255.0 (not the Class A default)
Network portion: Where mask = 255 → Octets 1, 2, and 3
Host portion: Where mask = 0 → Octet 4

Key Point:

Don't worry if the mask doesn't match the class default - always use the mask you're given in the assignment.

Quick Reference Tables

Network Portions by Class (Default Masks)

| Class | Network Octets | Example Address | Network Portion |
| --- | --- | --- | --- |
| A | 1 | 10.5.20.100 | 10 |
| B | 1, 2 | 172.16.5.50 | 172.16 |
| C | 1, 2, 3 | 192.168.1.25 | 192.168.1 |

Host Portions by Class (Default Masks)

| Class | Host Octets | Example Address | Host Portion |
| --- | --- | --- | --- |
| A | 2, 3, 4 | 10.5.20.100 | 5.20.100 |
| B | 3, 4 | 172.16.5.50 | 5.50 |
| C | 4 | 192.168.1.25 | 25 |

Assignment Types and Solutions

Type 1: Address Class Identification

Given: IP address
Find: Class letter (A, B, C, D, or E)
Method: Look at first octet only, use uppercase letters

Type 2: IPv4 Network Identification

Given: IP address (may include custom subnet mask)
Find: Which octets are network portion
Method: Find 255s in the subnet mask

Type 3: IPv4 Host Identification

Given: IP address (may include custom subnet mask)
Find: Which octets are host portion
Method: Find 0s in the subnet mask

Type 4: Default Subnet Mask Identification

Given: IP address
Find: Complete default subnet mask
Method: Identify class, look up default mask

Common Mistakes to Avoid

❌ Critical Errors:

- Forgetting dots in IP addresses and subnet masks

- Looking at all octets instead of just the first octet for class identification

- Not reading requirements carefully (network vs host)

- Typing errors: 225 instead of 255, missing dots

- Using lowercase letters for class identification

✅ Success Tips:

- Always include dots in your answers

- Read assignment requirements twice before starting

- Use capital letters for class identification (A, B, C)

- Utilize reference documents until concepts are memorized

- Double-check first octet class identification

- Write out the subnet mask on paper if needed for clarity

Memory Techniques

For Class Ranges:

- Class A: "All small numbers: 1-127"

- Class B: "Big jump: 128-191"

- Class C: "Close to 200: 192-223"

For Default Masks:

- Class A: "A = one 255" (255.0.0.0)

- Class B: "B = two 255s" (255.255.0.0)

- Class C: "C = three 255s" (255.255.255.0)

For Network vs Host:

"255 means Network, 0 means Host"

Practice Problems

Class Identification:

- 15.30.45.60 = ?

- 180.25.100.5 = ?

- 200.1.1.1 = ?

Network Identification:

- Address: 25.50.75.100, Find: Network octets

- Address: 155.10.20.30, Find: Network octets

- Address: 195.168.1.50, Find: Network octets

Host Identification:

- Address: 8.8.8.8, Find: Host octets

- Address: 140.100.50.25, Find: Host octets

- Address: 210.10.5.100, Find: Host octets

Default Mask Identification:

- 75.25.50.100 = ?

- 165.10.1.50 = ?

- 205.100.25.75 = ?

Why This Matters

For Network Administration:

- Proper subnet design and implementation

- Troubleshooting connectivity issues

- Security configuration and access control

For Certification Exams:

- CCNA requires manual calculation skills

- Network+ tests IPv4 fundamentals

- Many exams provide only pen and paper

Foundation for Advanced Topics:

- Subnetting and VLSM calculations

- Route summarization

- Network security implementations

Master these identification skills - they're essential before moving to subnetting calculations and advanced networking concepts.