Custom Subnetting and Subnet Mask Design Guide

Overview

This guide covers custom subnetting - the process of designing networks by borrowing bits from the host portion to create subnets. This is essential for network design and troubleshooting.

Why Learn Manual Subnetting?

Critical Reasons:

- Troubleshooting: Subnet calculators don't troubleshoot network problems

- CCNA Certification: Exams provide only pen, paper, and whiteboard - no calculators

- Understanding: Know why a PC can't communicate on the network

- Problem Solving: Identify incorrect IP addresses and configuration errors

The Two Essential Formulas

Formula 1: Subnets (Networks)

2^s = Number of Subnets

- s = Number of subnet bits borrowed

- Always round UP to next power of 2 if not exact match

- Having extra subnets provides scalability

Formula 2: Hosts

2^h - 2 = Usable Host Addresses

- h = Number of host bits remaining

- -2 accounts for network and broadcast addresses (first and last)

- NEVER forget the -2 - this is the most common mistake

Step-by-Step Subnetting Process

Step 1: Analyze the Requirements

Given Information:

- Base network address

- Number of subnets needed

- Number of usable hosts needed per subnet

Step 2: Basic Network Analysis

Determine Class and Default Mask

- Identify class from first octet

- Find default subnet mask

- Identify available bits for subnetting

Available Bits by Class:

- Class A: 24 host bits (octets 2, 3, 4)

- Class B: 16 host bits (octets 3, 4)

- Class C: 8 host bits (octet 4)

Step 3: Calculate Subnet Requirements

Subnet Calculation:

- Use formula: 2^s ≥ required subnets

- Find smallest power that meets or exceeds requirement

- Record the power number (s) = subnet bits needed

Host Calculation:

- Add 2 to required usable hosts (for network/broadcast)

- Use formula: 2^h ≥ total hosts needed

- Find smallest power that meets or exceeds requirement

- Record the power number (h) = host bits needed

Step 4: Verify Your Math

Critical Check: Subnet bits + Host bits = Available bits

- If they don't equal available bits, recalculate

- This validates your solution is possible

Worked Examples

Example 1: Class C Network

Given:

- Network: 192.168.1.0

- Required: 14 subnets, 14 usable hosts each

Solution Process:

Step 1: Network Analysis

- Class: C (192 in first octet)

- Default mask: 255.255.255.0

- Available bits: 8 (fourth octet only)

Step 2: Calculate Requirements

Subnets needed: 14

- 2^s ≥ 14

- 2^3 = 8 (too small)

- 2^4 = 16 ✓

- Subnet bits needed: 4

Hosts needed: 14 usable + 2 = 16 total

- 2^h ≥ 16

- 2^4 = 16 ✓

- Host bits needed: 4

Step 3: Verify Math

- Subnet bits: 4

- Host bits: 4

- Total: 4 + 4 = 8 ✓ (matches available)

Step 4: Results

- Subnet bits borrowed: 4

- Host bits remaining: 4

- Total subnets: 16 (2 extra for growth)

- Total addresses per subnet: 16

- Usable addresses per subnet: 14

Example 2: Class B Network

Given:

- Network: 172.16.0.0

- Required: 1024 subnets, 62 usable hosts each

Solution Process:

Step 1: Network Analysis

- Class: B (172 in first octet)

- Default mask: 255.255.0.0

- Available bits: 16 (third and fourth octets)

Step 2: Calculate Requirements

Subnets needed: 1024

- 2^s ≥ 1024

- 2^10 = 1024 ✓

- Subnet bits needed: 10

Hosts needed: 62 usable + 2 = 64 total

- 2^h ≥ 64

- 2^6 = 64 ✓

- Host bits needed: 6

Step 3: Verify Math

- Subnet bits: 10

- Host bits: 6

- Total: 10 + 6 = 16 ✓ (matches available)

Step 4: Results

- Subnet bits borrowed: 10

- Host bits remaining: 6

- Total subnets: 1024 (exact match)

- Total addresses per subnet: 64

- Usable addresses per subnet: 62

Creating Custom Subnet Masks

Process:

- Start with default mask

- Convert borrowed bits to decimal

- Add to appropriate octet

Binary to Decimal Conversion:

Use the binary value line for borrowed bits:

128 | 64 | 32 | 16 | 8 | 4 | 2 | 1

Class C Example (4 bits borrowed):

- Default: 255.255.255.0

- Binary: 11111111.11111111.11111111.00000000

- After borrowing 4 bits: 11111111.11111111.11111111.11110000

- 4 ones = 128+64+32+16 = 240

- Custom mask: 255.255.255.240

Class B Example (10 bits borrowed):

- Default: 255.255.0.0

- After borrowing 10 bits: 255.255.11111111.11000000

- First 8 bits: 11111111 = 255

- Last 2 bits: 11000000 = 128+64 = 192

- Custom mask: 255.255.255.192

Custom Address Maps

Purpose:

Visual representation showing network, subnet, and host portions

Format:

- N = Network bits (from default mask)

- s = Subnet bits (borrowed bits)

- h = Host bits (remaining bits)

Rules:

- 8 identical letters between dots = Capital letter

- Less than 8 = lowercase letters

- No dots within an octet's representation

Examples:

Class C Map (4 subnet, 4 host bits):

Address Map: N.N.N.ssssHHHH

- Network: 3 full octets

- Subnet: 4 bits

- Host: 4 bits

Class B Map (10 subnet, 6 host bits):

Address Map: N.N.sssssssss.hhhhhhh

- Network: 2 full octets

- Subnet: 8 bits + 2 bits

- Host: 6 bits

Assignment Question Types

1. Address Class

Method: Look at first octet, determine A/B/C

2. Default Subnet Mask

Method: Use class to find default mask

3. Number of Subnet Bits

Answer: The power number from subnet calculation

4. Number of Host Bits

Answer: The power number from host calculation

5. Total Number of Subnets

Answer: 2^(subnet bits) - actual result, not requirement

6. Total Number of Addresses

Answer: 2^(host bits) - includes unusable addresses

7. Usable Addresses

Answer: Total addresses - 2

8. Custom Subnet Mask

Method: Convert borrowed bits to decimal, add to default

9. Custom Address Map

Method: Use N/s/h notation based on bit allocation

10. Total Network Bits

Answer: Default network bits + subnet bits borrowed

Common Mistakes to Avoid

❌ Critical Errors:

- Forgetting -2 in host calculations

- Not adding 2 when calculating total host requirements

- Incorrect bit math - subnet + host ≠ available bits

- Wrong binary conversion when creating custom masks

- Forgetting dots in subnet masks

- Using wrong address map notation (capitals vs lowercase)

✅ Success Strategies:

- Always verify subnet bits + host bits = available bits

- Remember the -2 for unusable addresses

- Use power of 2 reference sheet for quick lookup

- Write out binary if needed for complex calculations

- Double-check custom subnet mask decimal conversion

- Practice with different class networks

Power of 2 Reference

| Power | Result | Usable (Result-2) |
| --- | --- | --- |
| 2^1 | 2 | 0 |
| 2^2 | 4 | 2 |
| 2^3 | 8 | 6 |
| 2^4 | 16 | 14 |
| 2^5 | 32 | 30 |
| 2^6 | 64 | 62 |
| 2^7 | 128 | 126 |
| 2^8 | 256 | 254 |
| 2^9 | 512 | 510 |
| 2^10 | 1024 | 1022 |

Practice Problems

Class C Practice:

Network: 192.168.10.0
Requirements: 6 subnets, 25 usable hosts each

Class B Practice:

Network: 172.20.0.0
Requirements: 500 subnets, 100 usable hosts each

Class A Practice:

Network: 10.0.0.0
Requirements: 4000 subnets, 1000 usable hosts each

Final Notes

Key Principles:

- Subnetting is borrowing bits from host portion

- Always account for network and broadcast addresses (-2)

- Round up to next power of 2 for scalability

- Verify your math before finalizing answers

For Certification Success:

- Practice manual calculations without calculators

- Memorize binary value line and power of 2 table

- Understand the underlying concepts, not just the formulas

Master these concepts and you'll be able to design networks, troubleshoot problems, and pass certification exams with confidence.