Subnet Ranges and Class C Address Calculation Guide

Overview

This guide covers subnet range calculations - determining the start and end addresses of each subnet after completing the subnetting math. This is essential for network implementation and troubleshooting.

Understanding Subnet Components

Four Critical Addresses in Every Subnet:

- Subnet ID (Network Address) - First address, cannot be assigned to devices

- First Usable - First address you can assign to devices

- Last Usable - Last address you can assign to devices

- Broadcast Address - Last address, used for network communication

The -2 Rule Explained:

- Total addresses = 2^h (includes all addresses)

- Usable addresses = 2^h - 2 (excludes subnet ID and broadcast)

- Cannot use: First address (subnet ID) and last address (broadcast)

- Can use: Everything in between

Key Terminology

When Asked for "Subnet Range":

Format: First address TO last address (Subnet ID to Broadcast) Example: 192.168.1.0 TO 192.168.1.63

When Asked for "Usable Range" or "Assignable Addresses":

Format: First usable TO last usable Example: 192.168.1.1 TO 192.168.1.62

Formatting Requirements:

- Use "TO" (with spaces) - not dashes or other symbols

- Include all dots in IP addresses

- Write complete addresses (all four octets)

The Magic Number Method

Finding the Magic Number:

- Create binary representation of subnet mask

- Locate the magic line between subnet bits (1s) and host bits (0s)

- Magic number = first value to the LEFT of the magic line

Class C Example:

- Binary line: 128 | 64 | 32 | 16 | 8 | 4 | 2 | 1

- 2 subnet bits = 11000000

- Magic line between 64 and 32

- Magic number = 64

Step-by-Step Range Calculation

Class C Example: Complete Walkthrough

Given Information:

- Network: 192.168.1.0

- Requirement: 4 subnets

- Class: C (192 in first octet)

Step 1: Complete the Math

Subnet calculation:

- 2^s ≥ 4 → 2^2 = 4 ✓

- Subnet bits: 2

Host calculation:

- Available bits: 8 (Class C)

- Used for subnets: 2

- Remaining for hosts: 6

- Host addresses: 2^6 = 64

- Usable addresses: 64 - 2 = 62

Step 2: Create Custom Subnet Mask

- Default: 255.255.255.0

- Binary conversion: 2 bits = 11000000 = 128 + 64 = 192

- Custom mask: 255.255.255.192

Step 3: Find Magic Number

- Binary: 11000000

- Magic line between positions 64 and 32

- Magic number: 64

Step 4: Calculate All Subnet Ranges

Starting Point:

- First subnet ID: 192.168.1.0 (given network address)

Using Magic Number (Add 64 each time):

| Subnet | Subnet ID | Broadcast | First Usable | Last Usable |
| --- | --- | --- | --- | --- |
| 1st | 192.168.1.0 | 192.168.1.63 | 192.168.1.1 | 192.168.1.62 |
| 2nd | 192.168.1.64 | 192.168.1.127 | 192.168.1.65 | 192.168.1.126 |
| 3rd | 192.168.1.128 | 192.168.1.191 | 192.168.1.129 | 192.168.1.190 |
| 4th | 192.168.1.192 | 192.168.1.255 | 192.168.1.193 | 192.168.1.254 |

Calculation Process:

- First subnet ID: 0 (given)

- Second subnet ID: 0 + 64 = 64

- Third subnet ID: 64 + 64 = 128

- Fourth subnet ID: 128 + 64 = 192

- Next would be: 192 + 64 = 256 (impossible, so we stop)

Broadcast Addresses:

- Rule: Broadcast = Next subnet ID - 1

- Last subnet: When next ID would be 256, broadcast = 255

Assignment Question Types and Answers

Question Type 1: Specific Subnet ID

Example: "What is the subnet ID of the second subnet?" Answer: 192.168.1.64

Question Type 2: Specific Broadcast Address

Example: "What is the broadcast address of the fourth subnet?" Answer: 192.168.1.255

Question Type 3: Subnet Range

Example: "What is the subnet range of the first subnet?" Answer: 192.168.1.0 TO 192.168.1.63

Question Type 4: Assignable/Usable Addresses

Example: "What are the assignable addresses for the third subnet?" Answer: 192.168.1.129 TO 192.168.1.190

Available Bits by Class

Class A (Default: 255.0.0.0):

- Available for subnetting: 24 bits (octets 2, 3, 4)

- Cannot change: First octet

- Math occurs in: Octets 2, 3, and 4

Class B (Default: 255.255.0.0):

- Available for subnetting: 16 bits (octets 3, 4)

- Cannot change: First two octets

- Math occurs in: Octets 3 and 4

Class C (Default: 255.255.255.0):

- Available for subnetting: 8 bits (octet 4)

- Cannot change: First three octets

- Math occurs in: Octet 4 only

Why This Matters

Network Implementation:

- Device configuration: Know which IP addresses to assign

- Router configuration: Set up proper subnet routing

- DHCP pools: Configure correct address ranges

Troubleshooting:

- Connectivity issues: Verify devices are in correct subnets

- IP conflicts: Ensure addresses don't overlap between subnets

- Network segmentation: Understand traffic flow between subnets

Common Mistakes to Avoid

❌ Critical Errors:

- Wrong format: Using dashes instead of "TO" with spaces

- Missing dots: Forgetting periods in IP addresses

- Incomplete addresses: Writing partial addresses (192.168.1 instead of 192.168.1.0)

- Off-by-one errors: Miscalculating broadcast addresses

- Magic number mistakes: Using wrong value for calculations

✅ Success Tips:

- Double-check math: Subnet bits + host bits = available bits

- Verify magic number: Always confirm with binary line

- Test your ranges: Next subnet ID - 1 = current broadcast

- Use proper format: Include spaces around "TO"

- Complete addresses: Always write all four octets

Practice Problem

Given:

- Network: 192.168.10.0

- Requirements: 8 subnets needed

Questions to Solve:

- What is the custom subnet mask?

- What is the magic number?

- What is the subnet range of the 5th subnet?

- What are the usable addresses for the 3rd subnet?

- What is the broadcast address of the 7th subnet?

Solution Process:

- Determine class: C (192 in first octet)

- Calculate subnet bits: 2^s ≥ 8 → 2^3 = 8, so s = 3

- Calculate host bits: 8 - 3 = 5 bits remaining

- Find magic number: 3 bits = 11100000, magic number = 32

- Calculate ranges: Start at 0, add 32 each time

Advanced Notes

For Class B and A Networks:

- Magic number calculation becomes more complex

- May need to write binary lines multiple times

- Spanning octets requires careful tracking

- Larger address spaces mean more subnets possible

Memory Aids:

- "Subnet ID + 1 = First Usable"

- "Broadcast - 1 = Last Usable"

- "Magic number = increment between subnet IDs"

- "When next ID = 256, you're done (Class C)"

Master subnet range calculations to implement networks correctly and troubleshoot connectivity issues effectively. This skill is essential for network administration and certification exams.